Lab 4.3.5  CDP neighbors

Estimated time: 30 min.

Objectives:

- Use CDP commands to get information about neighboring networks and routers.

- Display information on how CDP is configured for its advertisement and discovery frame transmission.

- Display CDP updates received on the local router.

Background:

In this lab you will use the show cdp command. Cisco Discovery Protocol (CDP) discovers and shows information about directly connected Cisco devices (routers and switches). CDP is a Cisco proprietary protocol that runs at the data link layer (layer 2) of the OSI model. This allows devices that may be running different network layer 3 protocols such as IP or IPX to learn about each other. CDP begins automatically upon a device's system startup, however if you are using Cisco IOS Release 10.3 or newer version of IOS you must enable it on each of the device's interfaces by using the cdp enable command. Using the command show cdp interface you will gather information CDP uses for its advertisement and discovery frame transmission. Use show cdp neighbors and show cdp neighbors detail to display the CDP updates received on the local router.

Tools / Preparation:

Prior to starting the lab you will need to connect a PC w/ HyperTerminal to a router using the router's console Interface with a roll-over cable. Work individually or in teams. Before beginning this lab you may want to read the Networking Academy First Year Companion Guide, Chapter 13. You should also review On-line Chapter 4. Be familiar with the following show commands:

- show interface

- show cdp

- show cdp interface

- show cdp neighbors

- show cdp neighbors detail

Resources Required:

- PC with Windows operating system and HyperTerminal installed

- Router connected to the PC with a console roll-over cable

- At least 3 routers interconnected via Ethernet or WAN simulation cables

Websites Sites Resources:

Routing basics 
General information on routers
2500 series routers 
1600 series routers
Terms and acronyms 
IP routing protocol IOS command summary
Beginning IP for new users

Notes:

Step 1 - Log on to router.

Explanation: Connect to the router and login. Enter the password cisco if prompted.

Step 2 - Gather information about the router you logged into by issuing the show interface command.

Task: Enter show interface command at the router prompt. 
Explanation: The router shows information about the configured interfaces.

1. Document the following information about the router:

a. What is the name of the router?

b. List IP address and subnet mask of the interfaces.

| Interface | IP Address | Subnet mask |
| --- | --- | --- |

c. List operational status of each interface.

| Interface | Interface Up or Down? (Carrier Detect Signal) | Line Protocol Up/Down? (Keep Alives Being received) |
| --- | --- | --- |

Step 3 - Display the values of the CDP timers, the interface status, and encapsulation used.

Task: Enter show cdp interface command at the router prompt. 
Explanation: The router responds with CDP information on all interfaces that have CDP enabled.

Global CDP settings can be seen using the show cdp command by itself.

2. How often is the router sending CDP packets? 
 

3. What is the holdtime value?

Step 4 - Display the CDP updates received on the local router.

Task: Enter show cdp neighbors command at the router prompt. 
Explanation: The router will respond with information about its neighbors that have CDP enabled.

4. Fill in the following table:

| Device and Port ID | Local Interface | Hold Time | Capability | Platform |
| --- | --- | --- | --- | --- |

Step 5 - Display details about CDP updates received on the local router.

Task: Enter show cdp neighbors detail from the router prompt.
Explanation: The router will display the entry address(es), IOS version, and  the same information as the show cdp neighbors command.

5.   Fill in the following table:

| Neighbor device name |  |  |  |
| --- | --- | --- | --- |
| Neighbor device type |  |  |  |
| IP address of interface attached to your router |  |  |  |
| Port ID of your router that the neighbor is on |  |  |  |
| Port ID of neighbor router that your router is on |  |  |  |
| IOS version of neighbor router |  |  |  |

Step 6 – Telnet to your neighbor router and issue show cdp neighbor.

Task:

a.      Telnet to neighboring router by entering telnet (hostname of router or IP address).
b.      Enter the password cisco.
c.      Enter show cdp neighbor at the router prompt you have telneted to.

Explanation: The router will respond with information about its neighbors that have CDP enabled. 
NOTE: Perform this step at router lab-b, lab-c, or lab-d and telnet to your two neighbors on either side.

6. Fill in the following tables:

First neighbor

| Device and Port ID | Local Interface | Hold Time | Capability | Platform |
| --- | --- | --- | --- | --- |

Second neighbor

| Device and Port ID | Local Interface | Hold Time | Capability | Platform |
| --- | --- | --- | --- | --- |

Lab 4.3.5  CDP neighbors - Answers

1. Connect to the router and log in. Enter the password cisco if prompted. Gather information about the router you logged into by issuing the show interface command. Document the following information about the router:

a. What is the name of the router? Lab-c

b. List IP address and subnet mask of the interfaces.

| Interface | IP Address | Subnet mask |
| --- | --- | --- |
| Ethernet 0 | 223.8.151.1 | 24 bits (255.255.255.0) |
| Serial 0 | 204.204.7.1 | 24 bits (255.255.255.0) |
| Serial 1 | 199.6.13.2 | 24 bits (255.255.255.0) |

c. List Operational status of each interface.

| Interface | Interface Up or Down? (Carrier Detect Signal) | Line Protocol Up / Down? (Keep Alives being received) |
| --- | --- | --- |
| Ethernet 0 | Up | Up |
| Serial 0 | Up | Up |
| Serial 1 | Up | Up |

Results of show interface command (Output generated by a Cisco 1601 router)

lab-c>show interface Ethernet0 is up, line protocol is up Hardware is QUICC Ethernet, address is 0060.474f.9bbd (bia 0060.474f.9bbd) Internet address is 223.8.151.1/24 MTU 1500 bytes, BW 10000 Kbit, DLY 1000 usec, rely 255/255, load 1/255 Encapsulation ARPA, loopback not set, keepalive set (10 sec) ARP type: ARPA, ARP Timeout 04:00:00 Last input 2d02h, output 00:00:02, output hang never Last clearing of "show interface" counters never Queueing strategy: fifo Output queue 0/40, 0 drops; input queue 0/75, 0 drops 5 minute input rate 0 bits/sec, 0 packets/sec 5 minute output rate 0 bits/sec, 0 packets/sec 7 packets input, 570 bytes, 0 no buffer Received 7 broadcasts, 0 runts, 0 giants 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort 0 input packets with dribble condition detected 100196 packets output, 10049637 bytes, 0 underruns 0 output errors, 0 collisions, 1 interface resets 0 babbles, 0 late collision, 0 deferred 0 lost carrier, 0 no carrier 0 output buffer failures, 0 output buffers swapped out Serial0 is up, line protocol is up Hardware is QUICC Serial Internet address is 204.204.7.1/24 MTU 1500 bytes, BW 56 Kbit, DLY 20000 usec, rely 255/255, load 1/255 Encapsulation HDLC, loopback not set, keepalive set (10 sec) Last input 00:00:08, output 00:00:04, output hang never Last clearing of "show interface" counters never Input queue: 0/75/0 (size/max/drops); Total output drops: 0 Queueing strategy: weighted fair Output queue: 0/64/0 (size/threshold/drops) Conversations 0/1 (active/max active) Reserved Conversations 0/0 (allocated/max allocated) 5 minute input rate 0 bits/sec, 0 packets/sec 5 minute output rate 0 bits/sec, 0 packets/sec 129492 packets input, 8117948 bytes, 0 no buffer Received 126687 broadcasts, 0 runts, 0 giants 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort 103169 packets output, 6985930 bytes, 0 underruns 0 output errors, 0 collisions, 7 interface resets 0 output buffer failures, 0 output buffers swapped out 18 carrier transitions DCD=up DSR=up DTR=up RTS=up CTS=up Serial1 is up, line protocol is up Hardware is QUICC Serial Internet address is 199.6.13.2/24 MTU 1500 bytes, BW 56 Kbit, DLY 20000 usec, rely 255/255, load 4/255 Encapsulation HDLC, loopback not set, keepalive set (10 sec) Last input 00:00:00, output 00:00:02, output hang never Last clearing of "show interface" counters never Input queue: 0/75/0 (size/max/drops); Total output drops: 0 Queueing strategy: weighted fair Output queue: 0/64/0 (size/threshold/drops) Conversations 0/3 (active/max active) Reserved Conversations 0/0 (allocated/max allocated) 5 minute input rate 1000 bits/sec, 2 packets/sec 5 minute output rate 1000 bits/sec, 2 packets/sec 107132 packets input, 5830048 bytes, 0 no buffer Received 102593 broadcasts, 0 runts, 0 giants 1 input errors, 0 CRC, 1 frame, 0 overrun, 0 ignored, 0 abort 104562 packets output, 6840784 bytes, 0 underruns 0 output errors, 0 collisions, 6 interface resets 0 output buffer failures, 0 output buffers swapped out 290 carrier transitions DCD=up DSR=up DTR=up RTS=up CTS=up

2. How often is the router sending CDP packets? Every 60 seconds

3. What is the hold time value? 180 seconds

Results of show cdp command (Output generated by a Cisco 2514 router)

lab-b>show cdp Global CDP information: Sending CDP packets every 60 seconds Sending a holdtime value of 180 seconds

4. Fill in the following table (answers may vary depending on setup of lab. In this case Lab-B is a 2514 and there is a Catalyst 1900 switch attached to the same Ethernet hub as Lab-B is attached to. Lab-A and Lab-C are Cisco 1600 models):

| Device and Port ID | Local Interface | Hold time | Capability | Platform |
| --- | --- | --- | --- | --- |
| Lab-c, Serial1 | Serial0 | 166 | Router | CPA1600 |
| Lab-a, Serial0 | Serial1 | 151 | Router | CPA1600 |
| 00C01D 81259B, Port 1 | Ethernet1 | 136 | Trans Bridge Switch | 1900 |

Results of show cdp neighbors command (Output generated by a Cisco 2514 router)

lab-b>show cdp neighbors Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge S - Switch, H - Host, I - IGMP Device ID; Local Intrfce; Holdtme; Capability; Platform; Port ID. lab-c; Ser 0; 166; R; CPA1600; Ser 1. lab-a; Ser 1; 151; R; 2500; Ser 0. 00C01D 81259B; Eth 1; 136; T S; 1900; 1. (Note: The last entry is the MAC address of a Cisco Catalyst 1900 Ethernet Switch)

5. Fill in the following table:

| Neighbor device name | Lab-c | Lab-a | 00C01D81259B |
| --- | --- | --- | --- |
| Neighbor device type | Cisco CPA1600 | Cisco CPA1600 | Cisco 1900 switch |
| IP address of interface attached to your router | 199.6.13.2 | 201.100.11.1 | 205.7.5.2 |
| Port ID of your router that the neighbor is on | Serial0 | Serial1 | Ethernet1 |
| Port ID of neighbor router that your router is on. | Serial1 | Serial0 | Port1 |
| IOS version of neighbor router | 11.1(7)AA | 11.1(7)AA | V5.35 |

Results of show cdp neighbors detail command (Output generated by a Cisco 2514 router)

lab-b>show cdp neighbors detail Device ID: lab-c Entry address(es): IP address: 199.6.13.2 Platform: cisco CPA1600, Capabilities: Router Interface: Serial0, Port ID (outgoing port): Serial1 Holdtime : 142 sec Version : Cisco Internetwork Operating System Software IOS (tm) 1600 Software (C1600-NSY-L), Version 11.1(7)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc2) Copyright (c) 1986-1996 by cisco Systems, Inc. Compiled Thu 24-Oct-96 05:24 by kuong Device ID: lab-a Entry address(es): IP address: 201.100.11.1 Platform: cisco CPA1600, Capabilities: Router Interface: Serial1, Port ID (outgoing port): Serial0 Holdtime : 173 sec Version : Cisco Internetwork Operating System Software IOS (tm) 1600 Software (C1600-NSY-L), Version 11.1(7)AA, EARLY DEPLOYMENT RELEASE SOFTWARE (fc2) Copyright (c) 1986-1996 by cisco Systems, Inc. Compiled Thu 24-Oct-96 05:24 by kuong Device ID: 00C01D 81259B Entry address(es): IP address: 205.7.5.2 Platform: cisco 1900, Capabilities: Trans-Bridge Switch Interface: Ethernet1, Port ID (outgoing port): 1 Holdtime : 132 sec Version : V5.35

6. Fill in the following tables:

First neighbor

| Device and Port ID | Local Interface | Hold time | Capability | Platform |
| --- | --- | --- | --- | --- |
| Lab-b, Serial1 | Serial0 | 142 | Router | CPA2500 |

Results of telnet and show cdp neighbors command. (Output from a Cisco 1604 router)

Lab-b>telnet lab-a Trying LAB-A (201.100.11.1)… Open User Access Verification Password: lab-a>show cdp neighbors Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge S - Switch, H - Host, I - IGMP Device ID; Local Intrfce; Holdtme; Capability; Platform; Port ID. lab-b; Ser 0; 142; R; CPA2500; Ser 1.

Second neighbor

| Device and Port ID | Local Interface | Hold time | Capability | Platform |
| --- | --- | --- | --- | --- |
| Lab-b, Serial0 | Serial1 | 142 | Router | CPA2500 |
| Lab-d, Serial1 | Serial0 | 151 | Router | CPA1600 |

Results of telnet and show cdp neighbors command. (Output from a Cisco 1601 router)

Lab-b>telnet lab-c Trying LAB-C (199.6.13.2)… Open User Access Verification Password: lab-c>show cdp neighbors Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge S - Switch, H - Host, I - IGMP Device ID; Local Intrfce; Holdtme; Capability; Platform; Port ID. lab-b; Ser 1; 142; R; CPA2500; Ser 0. lab-d; Ser 0; 151; R; CPA1600; Ser 1.